# Setting Up a Packet/Internet Gateway

Warren  Toomey  VK1XWT  --  April  1994Email:   WKT@CSADFA.CS.ADFA.OZ.AUAbstractThis paper discusses the reasons for setting up an amateur packet radio to Internet gateway, technical and hardware features of such a gateway, and security aspects of maintaining a gateway.Several sections of this paper require some knowledge of amateur packet radio operation,  Internet site operation,  connecting hardware,  and Internet security.  These sections will be marked,  and I will try to summarize each section in a manner that does not require this knowledge.This paper is intended for people who have to install a packet-Internet gateway, people who must maintain a packet-Internet gateway (both at the software and hardware level), people who manage a set of Internet machines, and people who provide the facilities for a set of Internet machines (such as a University).  This is a wide range of readers, so I apologize in advance if you find some of the material too far below or above your level of  understanding.

1     IntroductionThe  Internet  is  a  large  network  of  academic  and  commercial  computers in  the  United  States  that  provide  a  number  of  communication  services (such  as  electronic  mail  and  file  transfers)  to  its  users,  via the  TCP/IP  network  protocols.   This  network  is  connected  to  other networks,  such  as  the  AARNet  in  Australia,  and  networks  in  Europe and  Japan,  to  provide  a  global  computer  network. (known  as  amateurs  or  Hams)  throughout  the  world  who  use  radio communications  to  pass  messages,  chat,  do  research  into  aspects of  radio  communications  and  exchange  information  with  other  amateurs on  several  radio  frequencies  that  are  allocated  for  their  use. To  become  an  amateur,  an  individual  must  undergo  an  examination which  tests  the  individual's  knowledge  of  the  technical  operation of  radio  equipment,  radio  propagation,  electronic  theory,  as  well as  legal  and  other  aspects  of  amateur  radio.  Thus  amateurs  form a  highly  technical,  capable  and  responsible  group  of  people,  who are  able  to  provide  communication  during  emergencies,  natural  disasters, as  well  as  for  their  own  use.The  amateur  radio  service  is  also  actively  involved  in  finding new  and  more  efficient  forms  of  radio  communication  -  techniques such  as  single  sideband  radio  and  reduced  spectrum  radio  were  invented by  amateurs.   In  the  last  decade  or  so,  amateur  radio  operators have  been  involved  in  researching  new  forms  of  digital  radio  communications. This  research  has  led  several  amateurs  to  experiment  with  different communication  protocols  for  digital  radio  communications,  including the  TCP/IP  protocols  used  on  the  Internet  (see  Appendix  A).

The  Internet  protocols  have  been  found  to  be  very  useful  for radio  networking,  and  are  now  used  widely  by  amateurs.   To  this end,  a  block  of  16,387,064  Internet   addresses  has  been  allocated to  the  worldwide  amateur  radio  service.Fast  and  effective  digital  radio  communications  is  limited  over large  distances,  due  to  the  distortion  and  noise  introduced  by the  atmosphere,  and  the  curvature  of  the  Earth.   Amateurs  have sought  several  methods  to  overcome  these  limitations,  such  as  amateur satellites,  which  are  designed  and  built  by  amateurs,  and  provide store  and  forward  message  capabilities  as  well  as  digital  repeater services.Another  method  of  overcoming  the  distance  limitation  is  to  use the  Internet  itself  to  route  digital  messages  between  distant  amateur stations.   This  is  easy  to  accomplish  as  amateurs  are  able  to  use the  same  network  protocols,  TCP/IP,  as  used  on  the  Internet.   This method  of  long-distance  amateur  communication  has  been  implemented at  several  places  in  the  world:   Australia,  United States,  Switzerland, the  Netherlands  and  Canada,  and  has  been  found  to  be  very  simple to  set  up  and  operate,  and  is  very  reliable.

There  are  two  legal/policy  aspects  of  these  amateur  radio--Internet gateways.   The  first  is  the  amateur  radio  service  enforces  that amateur  communications  must  be  between  two  (or  more)  licensed  amateurs: therefore,  non-amateur  Internet  users  must  not  be  able  to  use  the gateways  to  transmit  messages.   The  second  is  that  many  AARNet/Internet sites'  policy  is  to  prevent  non-registered  users  from  accessing local/Internet  resources:   therefore  amateurs  may  only  pass  messages to  other  amateur  gateways.   The  software  that  controls  the  packet--Internet gateways  has  been  written  to  solidly  enforce  these  two  conditions.2     The  GatewayA  packet--Internet  gateway  obviously  must  be  able  to  communicate to  both  the  Internet,  and  the  amateurs  in  its  local  area.   Therefore, it  must  have  at  least  two  interfaces,  one  to  the  Internet,  and one  to  the  amateur  community.   This  is  shown  in  Figure  1.

µ §Figure  1:   Packet--Internet  Gateway  Block  DesignThe  gateway  is  a  machine  that  runs  the  software  that  enables it  to  perform  its  function.   Currently,  this  means  that  it  must be  an  IBM  PC  computer  or  clone,  running  the  latest  version  of  the NOS  software  by  Phil  Karn,  Gerard  van  der  Grinten,  and  other  flavors. The  software  itself  is  freely  available,  but  is  copyright  by  the software  authors.   For  performance  reasons,  I  would  recommend  a computer  with  at  least  the  performance  of  a  10MHz  IBM  XT,  with 640K  or  memory.   An  AT  of  course  would  be  better.   If  you  plan on  providing  file  transfer  or  mail  access,  a  hard  disk  is  needed.Each  interface  must  have  a  unique  IP  address,  so  that  both  the local  amateurs  and  the  other  gateways  can  communicate  with  it. The  Internet  interface's  address  is  assigned  by  the  AARNet/Internet administrator  of  the  institution,  and  the  amateur  radio  interface's address  is  assigned  by  the  local  amateur  IP  administrator.Each  interface  must  also  have  the  appropriate  hardware  needed for  it  to  communicate  with  its  destination.   On  the  amateur  radio side,  a  Terminal  Node  Controller,  a  transceiver  and  an  antenna convert  the  packets  from  the  gateway  into  radio  transmissions  that can  be  received  by  amateur  stations.   On  the  Internet  side,  there are  two  main  options:   an  Ethernet  card  connects  the  gateway  to the  local  area  network,  which  is  part  of  the  AARNet/Internet,  or a  serial  line  connects  the  gateway  to  another  machine  in  the  institution which  is  on  the  local  area  network,  and  hence  is  part  of  the  AARNet/Internet. The  serial  line  can  run  either  the  SLIP  or  PPP  protocol  to  communicate with  the  other  machine.That  completes  the  overview  of  the  hardware  setup  of  the  gateway. This  leads  on  to  several  considerations:

• The  gateway  and  other  hardware  must  be  reliable,  and  be  readily available  to  the  maintainer  of  the  gateway  to  fix  unforeseen problems. • The  gateway  must  be  placed  in  a  physical  position  to  suit  both its  connection  to  the  AARNet/Internet,  and  to  the  TNC/transceiver.• The  transceiver  and  antenna  must  be  in  a  position  to  satisfy both  the  needs  of  the  local  amateur  community,  and  the  wishes of  the  institution  where  they  are  to  be  situated.• If  using  Ethernet,  the  gateway's  position  must  allow  it  to be  physically connected  to  the  local  area  network.

At  this  point,  it  should  be  mentioned  that  at  least  one  of  the people  maintaining  the  gateway  must  have  a  current  amateur  radio license.   The  institution  providing  the  Internet  connection  may also  wish  to  appoint  another  person  to  ensure  the  gateway  meets the  policies  of  the  institution;  this  of  course  may  be  the  same person.3     How  the  Gateway  WorksAt  an  elementary  level,  the  gateway  works  as  follows:   Messages are  broken  up  into  packets  when  they  are  sent  over  the  Internet. When  the  gateway  receives  a  packet  on  one  interface,  it  checks the  destination  IP  address  in  the  packet,  and  according  to  its table  of  routes,  it  re transmits  that  packet  on  the  interface  that will  send  the  packet  to  its  destination.   For  example,  a  packet from  the  local  amateur  radio  community  to  a  gateway  in  another country  will  be  received  by  the  local  gateway,  re transmitted  by the  gateway  on  its  Internet  interface,  and  will  travel  along  the Internet  to  the  destination  gateway,  where  it  will  be  received and  re transmitted  by  the  transceiver  at  that  gateway.

In  practice,  this  is  not  as  simple,  because  of  the  following reason.   Even  though  the  global  amateur  community  has  been  allocated a  block  of  valid  Internet  addresses,  none  of  the  Internet  machines in  the  world  know  how  to  send  packets  to  these  addresses.   This is  because  up  until  recently,  no  amateur  computers  were  connected to  the  Internet.The  easiest  solution  would  be  to  advertise  the  routes  to  these addresses  to  all  Internet  machines,  but  this  was  not  done  for  the following  reasons:

• Amateur  stations  are  not  as  geographically  fixed  as  most  Internet  computers,  as  their  computers  are  generally  smaller,  and  the  radio  medium  allows  amateurs  to  move  around  and  still  communicate with  each  other,  and• Advertising  amateur  computer  addresses  to  all  Internet  machines greatly  increases  the  possibility  of  non-amateurs  accessing amateur  transmitters.

### The  solution  adopted  was  to  encapsulate  amateur  packets  received by  the  gateways  inside  another  packet,  transmit  this  bigger  packet to  the  destination  gateway,  which  unwraps  the  packet  and  transmits the  contents  [Woodburn  et  al.   1991].   This  is  illustrated  in  Figure 2.µ §Figure  2:   Amateur  Packet  EncapsulationThe  first  packet  shown  is  the  packet  transmitted  by  an  amateur and  received  by  the  local  gateway.   At  this  level,  the  source  and destination  address  are  both  amateur  addresses.   The  local  gateway determines  which  remote  gateway  this  packet  must  be  passed  to, and  encapsulates  the  packet  inside  a  new  packet,  with  source  and destination  addresses  being  the  Internet  addresses  of  the  local and  remote  gateway.   The  packet  is  then  placed  on  the  Internet, where  it  is  passed  to  the  remote  gateway.     This  encapsulated  packet  (the  second  packet  in  Figure  2)  is  seen by  the  Internet  to  have  a  valid  source  and  destination  address, and  can  be  passed  to  the  remote  gateway.   The  Internet  sees  the wrapped  packet  as  data,  which  it  ignores.At  the  remote  gateway,  the  packet  is  received  and  unwrapped; this  is  the  third  packet  in  Figure  2.   The  remote  gateway  determines which  interface  to  transmit  the  unwrapped  packet,  according  to the  destination  address.   Reply  messages  are  encapsulated  in  exactly the  same  manner.4     Security  Aspects  of  EncapsulationThe  encapsulation  method  satisfies  the  security  considerations discussed  in  the  Introduction:• Because  no  Internet  machines  know  how  to  route  packets  to  amateur gateways,  there  is  no  possibility  that  non-amateur  packets will  be  transmitted  on  amateur  transmitters.• All  incoming  amateur  packets  are  encapsulated  by  the  local gateway,  preventing  amateurs  from  accessing  any  Internet  machines, except  for  remote  gateways.   Therefore,  amateurs  see  the  connection between  the  gateways  as  a  very  long  piece  of  wire. There  are,  however,  other  ways  of  overcoming  these  barriers, by  using  some  of  the  applications  built  into  the  gateway  software; these  can  be  easily  prevented,  and  are  discussed  in  a  later  section.5     AX.25  EncapsulationOne  of  the  first  digital  protocols  used  by  amateurs,  and  still the  most  widely  used,  is  AX.25,  a  reliable  protocol  based  on  X.25. One  of  its  limitations  is  that  AX.25  packets  cannot  pass  through more  that  7  intermediate  machines  before  reaching  their  destination. This  limits  the  distance  AX.25  packets  can  travel.The  latest  versions  of  NOS  include  AX.25  encapsulation,  which operates  in  exactly  the  same  way  as  normal  IP  encapsulation,  thereby allowing  long  distance  AX.25  communication  while  still  obeying the  7  step  limit  [Kantor  1991].6     Preparing  the  Gateway  MachineThis  section  discusses  the  aspects  of  preparing  the  gateway  for the  software  that  will  make  the  gateway  work.   It  is  assumed  that:• You  have  a  fairly  good  idea  of  the  Internet  and  how  it  works.• You  have  a  reasonable  understanding  of  the  NOS  software.   If you  don't,  you  can  either  obtain  a  `plug  'n  play'  disk  from me  by  mailing  me  a  note  to  my  email  address  at  the  beginning of  the  paper,  or  you  can  anonymous  ftp  to  the  machine  MINNIE.CS.ADFA.OZ.AU, CD  to  hamradio/packet/plugplay,  and  get  the  software  from  there.• You  have  a  copy  of  the  NOS  software.   At  the  time  of  writing, the  NOS  flavor  most  used  on  gateways  is  Johan  Reinalda's  JNOS, and  the  current  version  is  1.07.   Other  NOS  versions  used  are Phil  Karn's  KA9Q,  and  Gerard  van  der  Grinten's  PA0GRI.  The latest  version  of  these  NOS  flavors  can  be  obtained  from  UCSD.EDU or  GRIVEL.UNE.OZ.AU.• You  have  a  gateway  that  meets  the  hardware  specifications  given in  the  Introduction,  and  the  hardware  needed  to  form  the  interfaces --  that  is,  a  TNC,  a  transceiver,  an  antenna,  and  an  Ethernet board  or  a  serial  link  to  a  machine  on  the  Internet  that  can talk  SLIP  or  PPP.  The  gateway  will  also  need  around  1  Megabyte of  free  disk  space  if  it  is  to  provide  file  transfer  or  mail access.• You  have  a  local  amateur  IP  address  for  the  machine,  and  a local  Internet  IP  address  for  the  machine.Your  first  step  is  to  give  the  gateway  two  domain  names,  a  name for  the  other  gateways  to  know,  and  a  name  for  the  amateurs  to know.   Your  Internet  name  should  follow  any  rules  specified  for your  institution.   Your  amateur  domain  name  should  be  of  the  following form:   NAME.CALLSIGN.AMPR.ORG  where  the  callsign  is  the  callsign of  the  amateur  that  will  be  operating  the  gateway,  and  the  name is  whatever  name  you  want  to  call  the  machine.   An  example  of  Internet and  amateur  domain  names  is  MINNIE.CS.ADFA.OZ.AU  and  MINNIE.VK1XWT.AMPR.ORG.You  need  to  register  the  Internet  name  on  the  Domain  Name  Server for  your  local  area  network.   You  then  need  to  register  your  amateur domain  name  with  the  Domain  Name  Server  for  the  amateur  service. This  is  the  DNS  server  at  ucsd.edu.   To  do  this,  send  email  to the  following  address:   AMPRADDR@UCSD.EDU.   The  subject  line  should be  empty,  and  the  email  should  have  the following  line:

### name.callsign.		IN	A	amateur.radio.ip.address

### For  example,  the  line  I  sent  in  for  my  gateway  was:

### minnie.vk1xwt.	IN	A	44.136.7.129

Aside: For those who understand DNS records, UCSD.EDU will also accept CNAME and MX records.  Also, you can delete previous entries by sending the same line, with the last field on the line the word `delete'.  Additions and changes to the DNS at ucsd.edu usually take a few days to be entered, so make sure you get things right first.7     Installing  the  NOS  SoftwareInstalling  the  NOS  software  can  be  done  in  three  stages:

• Install  the  software,  and  set  the  gateway  up  as  a  local  amateur radio  station;  this  includes  setting  up  the  TNC,  transceiver and  antenna.   This  should  not  be  too  hard  to  do,  as  the  local amateur  community  can  provide  information  on  how  to  do  this.• Alter  the  setup  to  include  an  interface  to  the  AARNet/Internet, using  either  an  Ethernet  board,  or  a  serial  link  running  SLIP or  PPP.  You  will  need  an  appropriate  Ethernet  driver  if  you are  using  an  Ethernet  board  --  these  can  be  obtained  from  UCSD.EDU in  the  directory  hamradio/drivers.  Read  the  documentation that  comes  with  the  driver  package.• Finally,  set  up  encapsulated  routes  to  the  other  amateur  gateways, and  email  their  operators  so  that  they  will  set  up  routes  back to  your  gateway.   Use  the  `ping',  `telnet'  and  `ftp'  commands in  NOS  to  ensure  that  the  encapsulation  is  working.

### Example  setup  files  are  given  in  the  appendices  ,  and  are  currently in  use  at  the  MINNIE.CS.ADFA.OZ.AU  gateway.   This  section  will discuss  aspects  of  a  gateway's  setup  that  I  think  need  attention.     To  begin  with,  the  host name  of  the  gateway  should  be  the  gateway's amateur  domain  name,  as  it  will  be  used  mainly  by  amateurs.   Ensure that  the  gateway  has  a  valid  amateur  callsign.   Every  interface used  by  the  gateway  should  use  the  amateur  IP  address,  except  for the  single  interface  to  the  Internet,  which  should  use  the  Internet IP  address.   The  default  route  should  be  the  Internet  interface, as  badly  addressed  packets  will  not  affect  anybody  there.   Ensure that  routes  are  as  specific  as  possible:   for  example,  if  your local  amateur  address  are  of  the  form  44:136:0:x,  then  you  should add  a  route  to  44.136.0/24.7.1     Adding  Encapsulated  IP  RoutesAdding  the  encapsulated  IP  routes  to  other  gateways  is  very  easy, as  is  shown  in  the  example  setup  file  in  the  appendices.   Note that  you  must  specify  the  Internet  IP  address  of  the  gateway,   and not  the  amateur  IP  address.7.2     Adding  Encapsulated  AX.25  RoutesDue  to  the  implementation  aspects  of  AX.25  encapsulation,  each encapsulated  AX.25  route  appears  as  a  new  interface  under  NOS, and  as  such,  needs  to  have  a  new  SSID  (Station  Sequence  ID),  as well  as  a  new  interface  name.   For  example,  the  pseudo-interfaces used  by  the  MINNIE  gateway  have  SSIDs  VK1XWT-2,  VK1XWT-3,  VK1XWT- 4,  VK1XWT-5  and  VK1XWT-6.   These  are  different  than  the  callsign used  for  the  gateway,  VK1XWT-1.   According  to  the  AX.25  protocol, this  allows  up  to  15  pseudo-interfaces,  with  SSIDs  of  -1  to  -15, as  well  as  the  local  amateur  interface.     For  AX.25  encapsulation  to  work,  you  need  to  already  have  an encapsulated  IP  route  defined  to  the   destination  gateway,  and  vice versa.When  using  the  encapsulated  AX.25  service,  you  must  remember that  you  need  to  use  the  SSID  of  the  interface  you  want  to  connect through.   For  example,  to  connect  to  MINNIE  locally,  you would do:

### net>  connect  vk1xwt-1

### However,  to  connect  to  a  machine  in  Hawaii  through  MINNIE,  you need  to  use  MINNIE's  SSID  of  --4:

### net>  connect  ah6bw-1  vk1xwt-4

Similarly,  a  user  in  Hawaii  connects  to  VK1XWT-4,  but  connects through  VK1XWT-1.8     Security  Considerations  --  Packet  FilteringOne  obvious  problem  with  a  gateway  is  that  packets  from  Internet machines  may  escape  out  onto  amateur  frequencies.   This  problem is  usually  minimized  by  not  advertising  routes  to  amateur  frequencies to  Internet  machines.   However,  this  is  security  through  obscurity.New  versions  of  NOS  have  solved  this  problem  by  adding  packet filtering,  known  as  `IP  Access'.   With  IP  access,  the  gateway  can allow  or  refuse  to  transmit  a  packet  on  any  interface  according to  the  source  IP  address  in  the  packet.   Thus,  access  to  amateur frequencies  can  be  denied  to  all  packets  who  have  non-amateur  IP addresses,  i.e.  IP  addresses  that  are  not  of  the  form  44.X.Y.Z.IP  Access  is  described  more  completely  in  the  appendices  and in  the  NOS  documentation.9     Security  Considerations  --  ServicesIf  you  use  your  gateway  with  no  services  running,  you  can  guarantee the  security  of  your  gateway.   However,  with  several  services, the  mailbox,  SMTP,  RIP  and  the  converse  service,  your  security can  be  breached.   Fortunately,  these  services  can  be  secured  as described  below.9.1     RIPRIP  is  the  routing  protocol  that  is  used  to  pass  routing  information around  between  machines  on  the  Internet.   Strictly  speaking,  a packet--Internet  gateway  does  not  need  to  run  RIP,  as  all  of  its routes  are  statically  defined.   The  only  reason  for  a  gateway  to run  RIP  is  to  inform  and/or  learn  from  other  gateways  when  new gateways  become  available.If  you  think  that  using  RIP  is  a  good  idea,  you  must  ensure  that:• You  do  not  broadcast  your  routing  table  to  your  local  network; if  you  do,  non-amateur  machines  will  learn  about  routes  to the  amateur  gateways,  and  may  allow  non-amateurs  to  use  amateur transceivers.• You  refuse  all  RIP  broadcasts  from  your  local  network;  if  you do  not,  amateurs  will  learn  about  the  Internet  addresses  on your  local  network,  which  are  non-amateur  addresses.• You  only  send  your  routing  table  to  destination  amateur  gateways, as  they  are  the  only  machines  who  are  interested  in  your  routes.

I  would  recommend  not  running  RIP  at  all.9.2     The  MailboxThe  mailbox  service  can  be  accessed  in  two  ways:   by  a  local  amateur connecting  to  your  machine  using  the  AX.25  protocol,  or  by  an  amateur or  non-amateur  connecting  to  your  machine  by  the  Telnet  protocol.

The  mailbox  provides  features  that  you  do  not  want  amateur  or non-amateur  users  to  access:

• A  Telnet  capability  which  can  connect  to  both  amateur  and  non-amateur machines.• A  Gateway  capability  which  can  connect  using  AX.25  to  amateur machines.• A  Netrom  capability  which  can  connect  using  NETROM  to  amateur machines.• A  Remote  operation  capability  which  allows  a  user  to  change the  current  setup  of  the  gateway.

## Fortunately,  these  are  easily  disabled  by  including  an  entry in  the  ftpusers  file  for  the  user  `anonymous',  with  permissions `1',  no  password,  and  a  home  directory  the  directory  /pub.   Make sure  that  the  ftpusers  file  is  not  kept  in  the  pub  directory,  or any  of  its  sub directories.You  may  wish  to  add  other  users  entries,  so  that  you  can  remotely Telnet  to  the  gateway  and  modify  its  state.   These  entries  should have  a  password,  and  you  should  never  Telnet  to  the  gateway  from another  amateur  machine,  as  the  password  will  be  broadcast  over the  airwaves  for  other  people  to  hear.9.3     SMTPSMTP  is  the  another  way  to  bypass  security,  as  it  allows  non-amateurs to  send  electronic  mail  to  amateurs,  and  vice  versa,  and  SMTP  cannot be  easily  set  up  to  prevent  this.You  may  initially  have  SMTP  enabled,  and  monitor  the  use  of  email to  see  if  there  is  any  current  security  problems;  however,  this leaves  the  gateway  open  for  email  interchange  which  may  be  illegal.9.4     The  Converse  ServiceThe  converse  service  is  similar  to  a  DX-Cluster  or  an  Internet Relay  Chat:   local  users  login  in  to  the  converse  server,  which is  itself  connected  to  other  server,  and  the  text  sent  by  local users  is  passed  to  all  the  other  converse  server  users.Again,  this  is  a  security  problem,  especially  as  many  converse servers  are  run  on  amateur-only  machines.   Thus,  Internet  users could  log  into  one  converse  server,  and  them  transmit  text  via amateur  frequencies  to  amateur-only  machines.10      Security  Considerations  --  Connection  FilteringYou  will  be  relieved  to  know  that  in  new  versions  of  NOS  there is  connection  filtering  as  well  as  packet  filtering.   Connection filtering  allows  the  gateway  to  accept  or  refuse  a  connection  to a  single  or  range  of  services  according  to  the  IP  address  attempting the  connection.This  filtering,  known  as  `TCP  Access',  allows  insecure  services such  as  SMTP  and  the  converse  service  to  be  restricted  to  amateur-only IP  addresses,  i.e.  only  44.X.Y.Z  addresses.   Of  course,  if  you  believe that  other  services  are  insecure  for  one  reason  or  another,  you can  use  TCP  Access  to  deny  connections  to  them  as  well.   More  details on  TCP  Access  are  given  in  the  appendices  and  in  the  NOS  documentation.11      ConclusionTo  summarize,  the  packet--Internet  gateway  provides  fast  and  reliable long-distance  digital  amateur  radio  communication,  which  up  until recently  has  been  difficult  if  not  impossible.   The  use  of  TCP/IP protocols  allows  easy  connection  to  the  AARNet/Internet,  as  well as  the  local  amateur  community.   This  long-distance  connection not  only  provides  reliable  communication  for  amateur  and  emergency use,  but  gives  the  amateur  radio  community  another  avenue  for  research into  radio  communication  techniques.The gateway software provides the method to connect amateur gateways together,  while  at  the  same  time  ensures  the  gateway  can  be  operated correctly  from  both  the  amateur  and  non-amateur  point  of  view.I  have  operated  and  maintained  a  packet--Internet  gateway  in Canberra  since  April  1991,  and  have  found  it  to  be  very  reliable, very  easy  to  maintain,  and  very  secure.   Moreover,  due  to  the  communication speeds  used  in  the  amateur  community,  the  gateway  has  added  a  negligible burden  to  the  AARNet/Internet  use  of  my  local  network.I  would  be  glad  to  answer  any  questions  that  are  emailed  to  me at  the  Internet  mail  address  given  at  the  beginning  of  this  paper, although  I  cannot  guarantee  the  promptness  of  my  replies.   I  would also  greatly  appreciate  comments,  criticisms  and  suggestions  about this  paper  and  how  it  can  be  improved.                          Warren  Toomey  VK1XWT.

A     BibliographyThis  bibliography  includes  the  papers  describing  the  encapsulation protocols  used  by  a  packet  radio--Internet  gateway,  as  well  as some  example  papers  published  by  amateurs  in  the  field  of  packet radio.

## Diersing  et  al.   1989]  R.J.Diersing,  J.W.Ward.   Packet  Radio  in the  Amateur  Satellite  Service. IEEE  J  Selected  Areas  in  Communications, Vol.  7  No  2.   February  1989.[Enmore  1990]  G.  Enmore  N6GN.   Physical  Layer  Considerations  in Building  a  High  Speed  Amateur  Radio  Amateur  Network.   ARRL/CRRLAmateur  Radio  9th  Computer  Networking  Conference.   London,  Ontario. Canada.   September  1990.[Flaherty  1988]  P.  Flaherty  N9FZX.   Digital  Radio  Networks  and  Spectrum Management.   ARRL/CRRL  Amateur  Radio  7th  Computer  Networking  Conference. Columbia,  Maryland,  America.   October  1988.[Geier  et  al.   1990]  J.  Geier,  M.  DeSimio  WB8MPF  and  B.  Welsh  KD8WG. Network  Routing  Techniques  and  Their  Relevance  to  Packet  Radio Networks.   ARRL/CRRL  Amateur  Radio  9th  Computer  Networking  Conference. London,  Ontario.   Canada.   September  1990.[Ioannidis  et  al.   1991]  J.  Ioannidis,  D.  Duchamp  and  G.  Maguire Jr.   IP-Based  Protocols  for  Mobile  Networking.   ACM  SIGCOMM  '91 Conference.   Zurich,  Switzerland.   September,  1991.[Kahn  et  al.   1985]  P.R.Kahn,  H.E.Price,  R.J.Diersing.   Packet Radio  in  the  Amateur  Service.   IEEE  J  Selected  Areas  in  Communications, Vol  3,  No  5.   May  1985.[Kantor  1991]  B.  Kantor  WB6CYT.   Internet  Protocol  Encapsulation of  AX.25  Frames.   Request  for  Comments  memo  1226.   May  1991.[Karn  1990]  P.  Karn  KA9Q.   MACA  --  A  New  Channel  Access  Method  for Packet  Radio.   ARRL/CRRL  Amateur  Radio  9th  Computer  Networking Conference.   London,  Ontario.   Canada.   September  1990.[Neben  1983]  B.  Neben  K9BL.   Packet  Radio  for  Emergency  Communications. ARRL  Amateur  Radio  2nd  Computer  Networking  Conference.   San  Francisco, California.   March  1983.[Sproul  et  al.   1990]  M.  Sproul  KB2ICI  and  K.  Sproul  WU2Z.   Long Distance  Packet  Mail  via  Satellite.   ARRL/CRRL  Amateur  Radio  9th Computer  Networking  Conference.   London,  Ontario.   Canada.   September 1990.[Woodburn  et  al.   1991]  R.  Woodburn  and  D.  Mills.   A  Scheme  for an  Internet  Encapsulation  Protocol.   Request  for  Comments  memo 1241.   July  1991.

B     Existing  Packet--Internet  GatewaysFollowing  is  a  table  of  the  existing  gateways  as  at  the  13th  of May, 1993.

| AREA | OPERATOR | EMAIL ADDRESS |
| --- | --- | --- |
| Los Angeles, California | Paul Brewer, KI6CQ | pjb@hss.CalTech.edu |
| Southern California | Christopher Mierzwa, N2GFN | mierzwa@usc.edu |
| Hawaii | Antonio Querubin, AH6BW Univ. of Hawaii ARC | tony@mpg.phys.hawaii.edu |
| Oregon | Johan Reinalda, WG7J | johan@ece.orst.edu |
| Dallas/Fort Worth, Texas | Jack Snodgrass, KF5MG Univ. of North Texas RC | system200.cecs.unt.edu |
| Memphis, Tennesee | Suresh Kagoo, N9GSA | kagoos@memstvx1.memst.edu |
| Salt Lake City, Utah | Univ. Utah Amateur Radio Club | uugate.aim.utah.edu |
| Indiana | Dwight Hazen, K9IU | hazen@hazen.ucs.indiana.edu |
| New Jersey | NJ Institute of Technology ARC | cdp@heertz.njit.edu |
| Northern New York | Clarkston Amateur Radio Club | rob@sun.soe.clarkston.edu |
| Northern Illinois | Bob Van Valzah, KE9YQ | bob@ke9yq.imsa.edu |
| Central and Southern Illinois | Chuck Henderson, WB9UUS | chuck@bradley.bradley.edu |
| South Texas | Kurt Freiberger, WB5BBW Texas A&M Amateur Radio Club | kurt@cs.tamu.edu |
| Austin, Texas | Paulus Adisoemarta, N5SNN Univ. of Texas ARC | paulus@nextover.pe.utexas.edu |
| Michigan | Ron Atkinson, N8FOW | ron@chaos.eng.wayne.edu |
| Maine | Steve Goldsmith, N1JAD | steve@gandalf.umcs.maine.edu |
| North East Kansas | Chris Whittenburg, N0TWA Univ. of Kansas ARC | whitten@kuhub.cc.ukans.edu |
| Reno, Nevada | Bill Healy, N8KHN Univ. of Nevada at Reno ARC | healy@moriah.ee.unr.edu |
| South West Germany | Olaf Erb, DC1IK University Karlsruhe | erb@insu1.etec.uni-karlsruhe.de |
| Ottawa, Canada | Barry McLarnon, VE3JF | barry@dgbt.doc.ca |
| Waterloo, Canada | Roger Sanderson, VE3RKS | rsanders@sunee.uwaterloo.ca |
| Toronto, Canada | Michael Durrant, VE3PNX Ryerson Univ. Amateur Radio Club | spcl0006@ryerson.ca |
| AREA | OPERATOR | EMAIL ADDRESS |
| Winnipeg, Canada | Dan Keizer, VE4DRK | ve4drk@muug.mb.ca |
| Prince George, Canada | Lyndon Nerenberg, VE7TCP | lyndon@unbc.edu |
| Canberra, Austrailia | Carl Makin, VK1KCM | makin@hhcs.gov.au |
| Sydney, Austrailia | Univ. of Technology, Sydney ARS | radio.gw.uts.edu.au |
| Newcastle, Austrailia | Newcastle University | ccdrw@cc.newcastle.edu.au |
| Central West Austrailia | Adam Robertson, VK2XRA Charles Sturt Univ. Bathurst ARC | aroberts@csu.edu.au |
| Melborne Austrailia | Peter Hallgarten, VK3AVE | vk3ave@csource.oz.au |
| Queensland Austrailia | Queensland Univ. of Technology | joyce@vk4kiv.qut.edu.au |
| Rotterdam Netherlands | Univ. of Delft Amateur Radio Club | chris@duttnpc.tn.tudelft.nl |
| Stockholm Sweden | Rogan Clements, SM0KJD | rogan@math.kth.se |
| Poland | Andrezej Brandt, | pc132b.mimuw.edu.pl |
| Poland | Pawel Jalocha, SP9VRC | sp9vrc@192.86.14.25 |
| Santa Catarina, Brazil | Marcio Marchini, PU5CPJ | edugraf@brufsc.bitnet |

C     Example  NOS  Configuration  FilesThis  section  examines  a  set  of  example  configuration  files  for NOS  on  the  gateway  MINNIE.CS.ADFA.OZ.AU.   Note  that  not  all  gateways have  the  same  requirements  or  offer  the  same  services,  and  not all  use  the  same  variety  of  NOS.  Therefore,  take  the  examples  given here  with  a  guide  for  your  own  files.C.1     IntroductionIn  order  to  understand  the  configuration  files  presented  here, we  need  to  first  examine  the  purpose  of  the  gateway,  its  services, and  the  software  it  uses.MINNIE.CS.ADFA.OZ.AU  is  an  Internet-AMPRnet  gateway  which  routes packets  between  itself  and  other  gateways  via  encapsulation.   It also  offers  the  following  services:

• Anonymous  file  transfer  with  the  ftp  service.• Logins  via  AX.25  or  telnet  to  the  NOS  mailbox.• Mail  delivery  using  the  smtp  service.• Determination  of  the  current  users  of  the  gateway  with  the finger  service.• A  converse  service  that  is  the  TCP/IP  equivalent  to  a  DX-Cluster.

It  should  be  noted  that  the  mailbox  in  particular  provides  more services  than  just  sending  and  receiving  mail;  mailbox  users  may be  permitted  to  telnet  or  AX.25  connect  out  of  the  mailbox,  use the  finger  and  converse  services,  and  upload/download  files.   Ensuring reasonable  privileges  to  mailbox  users  will  be  dealt  with  later. The  most  important  service  is  that  of  encapsulating  AMPRnet  packets destined  for  other  gateways,  and  vice  versa.   Unlike  most  ordinary TCP/IP  hosts,  a  gateway  has  two  or  more  interfaces,  and  at  least two  IP  addresses.   MINNIE  has  the  following  addresses  and  domain names:

• MINNIE.CS.ADFA.OZ.AU  [131.236.20.70]• MINNIE.VK1XWT.AMPR.ORG  [44.136.7.129]

and  three  interfaces:

• An  ethernet  interface  (EC0)  for  Internet  users• A  KISS  AX.25  interface  (AX0)  for  AMPRnet  users,  and• A  pseudo-interface  (ENCAP)  which  performs  the  encapsulation.

With  the  encapsulation  correctly  set  up,  MINNIE  can  be  accessed by  all  Internet  users  as  MINNIE.CS.ADFA.OZ.AU,  and  by  all  AMPRnet users  as  MINNIE.VK1XWT.AMPR.ORG.Packet  encapsulation  is  a  two-way  process.   MINNIE  services  the range  of  AMPRnet  addresses  44.136.7.128  to  44.136.7.255,  i.e.  44.136.7.128/25.  Other  gateways  who  wish  to  route  packets  to  this  AMPRnet  address range  must  include  an  encapsulated  route  so  that  packets  will  be encapsulated  and  then  routed  to  minnie's  Internet  address,  i.e.

### route  addprivate  44.136.7.128/25  encap  minnie.cs.adfa.oz.au

Similarly,  minnie  must  have  a  corresponding  set  of  encapsulated routes  back

### ...    route  addprivate  dest`AMPRnet`range  encap  dest`gateway        ...

The  list  of  encapsulated  routes  to  all  known  gateways  is  available via  anonymous  FTP  from  MINNIE.CS.ADFA.OZ.AU  in  the  `gateways'  directory, as  the  file  `GATEWAYS.XXX',  where  `XXX'  are  digits.C.2     Booting  the  GatewayGateways  are  usually  run  as  turn-key  machines;  the  box  is  turned on  and  the  NOS  software  is  automatically  started.   Therefore,  in the  gateway's  boot  file  you  need  to  do  several  things:

• Set  up  some  environment  variables  used  by  NOS• Install  the  packet  drivers  you  need  for  NOS• Run  NOS

Here  is  an  example  autoexec.bat:

### @echo  offPATH  c:"bin;c:"ka9q"bin;remrem       Set  up  some  environment  variables  for  NOSremrem  set  TMP=C:"tmprem  set  TEMP=C:"tmpremrem       Set  up  miscellaneous  environment  variablesremset  USER=warrenset  TERM=nansiloadhigh  \ka9q\bin\wd8003e  0x70  -o  5  0x280  0xc800nos

On  MINNIE,  all  of  the  gateway  files  and  programs  are  kept  under the  KA9Q  directory.   The  programs  and  batch  files  are  kept  in  KA9Q\BIN. NOS  uses  the  TMP  or  TEMP  variable  to  determine  where  it  creates temporary  variables:   I  can't  remember  which  one  so  I  set  both.The  packet  driver  for  MINNIE's  WD8003E  card  is  then  loaded.   Each packet  driver  has  a  slightly  different  calling  convention:   here the  driver  is  loaded  to  use  software  interrupt  0x7e,  hardware  interrupt 5,  with  the  card's  registers  at  0x280  and  the  card's  buffer  memory at  0xc800.   Only  the  interrupt  numbers  are  of  interest  to  NOS.Finally,  a  batch  file  kept  in  KA9Q\BIN  called  NOS.BAT  is  executed.C.3     The  NOS  Batch  FileThe  NOS  batch  file  usually  runs  as  an  infinite  loop,  restarting NOS  is  ever  it  is  terminated:   this  is  useful  for  when  the  gateway must  run  unattended.   The  batch  file  cleans  up  the  lock  files  from the  mail  subsystem  (in  case  NOS  was  abruptly  terminated).   If  the gateway  has  logging  turned  on,  the  last  log  file  is  appended  to a  cumulative  log  file,  and  then  zeroed.   Thus  a  gateway  administrator can  look  in  nos.log  for  logging  information  since  the  last  NOS restart,  and  in  nos_old.log  for  the  remaining  log  information.

### @echo  off:Againcd  \ka9qset  TZ=EST14del  \ka9q\spool\mqueue\*.lckdel  \ka9q\spool\mail\*.lcktype  nos.log  >>  nos`old.logdel  nos.logcopy  dom.txt  domain.txtrem  \ka9q\bin\oldnos.exe  -d  /ka9q  minnie.net\ka9q\bin\jnos107b.exe  -f  minn107.cfggoto  Again

One  line  to  note  is  that  the  DOMAIN.TXT  file  (which  holds  TCP/IP name/address  pairs)  is  recreated  from  the  DOM.TXT  file.   This  is done  because  some  versions  of  NOS  (for  example  GRINOS),  would  rewrite the  file,  removing  any  comments.   This  ensures  that  an  original version  is  kept  intact.Finally,  the  NOS  program  is  started.   This  can  be  done  in  two ways.   With  some  NOS  flavors,  you  specify  the  directory  when  all the  configuration  files  are  kept,  and  the  NOS  initialization  file, e.g.

### \ka9q\bin\oldnos.exe  -d  /ka9q  minnie.net

With  these  flavors,  all  of  the  NOS  service  configuration  files are  kept  in  fixed  locations  in  the  NOS  directory,  which  can  be inflexible.   In  new  NOS  flavors,  e.g.  JNOS  1.07,  a  directory  configuration file  is  first  read  to  inform  NOS  of  the  location  of  the  NOS  service configuration  files,  e.g.

### \ka9q\bin\jnos107b.exe  -f  minn107.cfg

If  you  have  a  NOS  flavor  that  performs  this,  go  on  to  the  next section.   If  not,  skip  the  next  section.C.4     The  Directory  Configuration  FileAs  mentioned,  this  file  is  read  by  newer  versions  of  NOS  to  allow you  to  set  up  a  flexible  directory  system.   I  won't  attempt  to explain  every  line;  see  the  NOS  documentation  for  your  NOS  flavor for  more  information.   The  most  important  line  is  the  one  which tells  NOS  where  the  NOS  initialization  file  is  kept:

### Startup  =  /ka9q/minnie.net

Once  this  file  is  read,  NOS  read  the  initialization  file.   The rest  of  this  file  is  left  for  your  perusal.

### #SAMPLE:  these  are  the  defaults  used.#These are new names  for  the  various  files and directories used in  nos.#To  use  them,  run  nos  as  'nos  -fnos.cfg'#If  you  don't change a particular filename, you should comment out  each#unneeded  line,  since  they  allocate  memory  for  the  new  name.#lines  need  to  be  either  comments  (starting  with  #)#or  have  a  valid  'file=filename'  format.#(lines  are  NOT  case  sensitive.)#all  others  are  ignored;  this  allows  for  different  compiles  to#use  the  same  files-configuration  file...#both  spaces  or  tabs  can  be  used  as  separators.#921125  -  WG7J#the  autoexec  file  containing  system  setupStartup  =  /ka9q/minnie.107#the  user  permission  fileUserfile  =  /ka9q/ftpusers#the  ftp  host  file  for  auto-login#Hostfile  =  /net.rc#the  mail  log  fileMaillog  =  /ka9q/spool/mail.log#the  directory  where  local  mail  gets  deliveredMailspool  =  /ka9q/spool/mail#the directory where mail  gets  queued  for  the  smtp  daemon  to  handleMailqdir  =  /ka9q/spool/mqueue#this  should  have  same  path  as  the  above!!Mailqueue  =  /ka9q/spool/mqueue/*.wrk#if  you  route  mail,  here  is  goes#Routeqdir  =  /ka9q/spool/rqueue#the  mail  alias  fileAlias  =  /ka9q/alias#the  domain.txt  fileDfile  =  /ka9q/domain.txt#directory  where  finger  files  goFdir  =  /ka9q/finger#the  file  where  the  finger  database  is  maintainedFdbase  =  /ka9q/finger/dbase.dat#the  list  of  areas  on  the  system#Arealist  =  /ka9q/spool/areas#mailbox  message  of  the  dayMotdfile  =  /ka9q/spool/motd.txt#mail  rewrite  rulesRewritefile  =  /ka9q/spool/rewrite#user  signatures  go  here#Signature  =  /spool/signatur#Bulletin  ID's  go  here#Historyfile  =  /spool/history#Help  files  go  in  this  directoryHelpdir  =  /ka9q/spool/help#the  user  defaults  file  (created  by  system)UDefaults  =  /ka9q/spool/users.dat#backup  of  the  aboveUDefbak  =  /spool/ka9q/users.bak#Convers user info file; notice that default is the same  as  Fdbase  file  !Cinfo  =  /ka9q/finger/dbase.dat#pop  users  are  listed  in  this#Popusers  =  /popusers#FTP  message  of  the  dayFtpmotd  =  /ka9q/spool/ftpmotd.txt#NNTP  directoryNewsdir  =  /ka9q/spool/news#BBS  forward  file#Forwardfile  =  /spool/forward.bbs#saved  netrom  routes  go  here#Netromfile  =  /netrom.sav#these  commands  get  executed  on  exit#Onexit  =  /onexit.nos#expire  command  file#Expirefile  =  /spool/expire.dat#NNTP  active  file#Active  =  /spool/news/active#NNTP  pointer  file#Pointer  =  /spool/news/pointer#NNTP  info#NInfo  =  /spool/news/info#NNTP  help#Nhelp  =  /spool/news/help#NNTP  message  history  fileHistory  =  /ka9q/spool/news/history#NNTP  forward#Forward  =  /spool/news/forward#NNTP  poll#Poll  =  /spool/news/poll

C.5     The  Domain  FileBefore  going  on  to  look  at  the  NOS  initialization  file,  we  need to  first  examine  the  domain  file.   Although  NOS  can  use  name  servers to  look  up  TCP/IP  name/address  pairs  it  doesn't  know,  these  are not  used  while  processing  the  initialization  file.   Therefore  the domain  file  (usually  DOMAIN.TXT)  must  contain  all  of  the  TCP/IP names  used  by  the  initialization  file.   You  may  also  want  to  include names  that  you  use  frequently,  so  that  NOS  won't  have  to  query any  name  servers  to  resolve  them.   Note  however,  that  the  domain file  is  static,  and  you  may  need  to  keep  it  up  to  date  periodically to  ensure  that  the  name/address  pairs  are  correct.   Here  is  MINNIE's DOMAIN.TXT:

#  Last  updated:  10:00am  921209##  Canberra  packet  stations#gw.vk1bud.ampr.org.		IN	A 	44.136.0.40minnie.vk1xwt.ampr.org.	IN	A	44.136.7.129##  Gateways  to  other  packet  stations###  ADFA  machines#cserve.cs.adfa.oz.au.		IN	A	131.236.20.1cs`gate.cs.adfa.oz.au.		IN	A 	131.236.20.2cspyr1.cs.adfa.oz.au.		IN	A	131.236.20.7joruth.cs.adfa.oz.au.		IN	A	131.236.20.69minnie.cs.adfa.oz.au.		IN	A	131.236.20.70convex.cc.adfa.oz.au.		IN	A	131.236.1.1ccadfa.cc.adfa.oz.au.		IN	A	131.236.1.2sserve.cc.adfa.oz.au.		IN	A	131.236.1.17###  Other  machines#munnari.oz.au.			IN	A 	128.250.1.21##  Aliases#minnie.cs.		IN	CNAME	minnie.cs.adfa.oz.au.csadfa.			IN	CNAME	csadfa.cs.adfa.oz.au.cspyr1.			IN	CNAME	cspyr1.cs.adfa.oz.au.ccadfa.			IN	CNAME	ccadfa.cc.adfa.oz.au.sserve.			IN	CNAME	sserve.cc.adfa.oz.au.cserve.			IN	CNAME	cserve.cs.adfa.oz.au.convex.		IN	CNAME	convex.cc.adfa.oz.au.minnie.			IN	CNAME	minnie.vk1xwt.ampr.org.C.6     The  NOS  Initialization  FileThe  NOS  initialization  file  is  where  the  gateway's  network  addresses and  interfaces  are  set  up,  and  where  the  services  on  the  gateway are  started.   Some  commands  must  occur  before  others;  in  general, I  recommend  performing  things  in  the  following  order:

• Define  the  gateway's  main  host name  and  IP  address.• Describe  the  memory  buffers  and  memory  parameters.• Specify  the  interfaces  used  and  their  soft/hardware  interrupts.• Specify  the  interfaces  addresses,  MTUs  and  broadcast  addresses.• Detail  what  domain  name  servers  are  to  be  used.• Set  up  routes  to  the  gateway's  interfaces,  and  to  other  gateways.• Set  up  TCP  and  IP  parameters.• Set  up  service-specific  parameters.• Start  the  gateway  services.• Any  miscellaneous  commands.

Once  the  initialization  file  is  finished,  NOS  falls  to  the  command line  prompt  as  usual.   For  the  rest  of  this  section,  we  will  look at  an  example  initialization  file,  MINNIE's  initialization  file, broken  into  the  parts  given  above. The  gateway's  domain  name  and  IP  address  is  given  first.   You must  choose  one  of  the  two  name/addresses  that  the  gateway  has. I  use  the  AMPRnet  name  so  as  not  to  confuse  the  amateur  users; the  Internet  users  can  cope.

#      A  U  T  O  E  X  E  C  .  N  E  T       F  O  R       W  G  7  J###         Local  Definitions#host name  minnie.vk1xwt.ampr.orgip  address  minnie.vk1xwt.ampr.orgThe  number  of  interrupt  buffers  should  be  set  so  as  to  minimize the  number  of  lost  packets,  especially  from  an  Ethernet.   MINNIE uses  10  interrupt  buffers.   Each  buffer  should  be  at  least  as  large as  the  maximum  packet  size;  the  default  buffer  size  is  2048  bytes.  MINNIE uses  the  `efficient'  form  of  memory  allocation  which  helps to  keep  the  amount  of  available  memory  high.

##         Memory  Parameters##  nibufs  -  number  of  interrupt  buffers#  eff  -  efficient.   It's  a  better  memory  allocation  algorithm.mem  nibufs  10mem  eff  on

The  interfaces  used  by  the  gateway  are  now  attached.   MINNIE has  one  Ethernet  interface  EC0  which  is  a  packet  driver,  and  a KISS  interface  ax0  using  the  built-in  NOS  KISS  driver.   Note  that the  soft/hardware  interrupt  values  used  by  the  packet  driver  must be  the  same  as  used  when  the  packet  driver  was  loaded  in  the  AUTOEXEC.BAT file.The  largest  packet  size  is  also  specified  on  the  attach  lines, 1500  for  the  ec0  interface,  and  576  for  the  AX0  interface.   You should  read  the  NOS  documentation  for  the  other  parameters.

##         Attach  ports##  We  have  2  interfaces:#         ec0  connects  minnie  to  the  ADFA  net#         ax0  connects  minnie  to  the  local  AMPR  subnetattach  packet  0x70  ec0  5  1500attach  asy  0x2f8  3  ax25  ax0  1024  576  9600

With  the  interfaces  attached,  their  addresses  etc.   must  now be  given.   A  gateway  has  two  other  interfaces:   loop back  is  used to  route  the  gateway's  packets  back  to  itself  (for  debugging  purposes), and  ENCAP  encapsulates  packets  and  passes  them  back  to  the  gateway for  further  routing.For  each  interface  you  should  specify  the  IP  address,  the  maximum packet  size  and  the  broadcast  address.   The  latter  should  have all  non-network  address  bits  as  1's.   For  example,  44.A/16  would have  broadcast  address  44.A.255.255,  and  44.A.B/24  would  have  broadcast address  44.A.B.255.   The  ENCAP  interface  should  have  the  gateway's AMPRnet  address,  but  broadcast  should  not  be  specified  (it  doesn't make  sense  anyway).

##         Interface  addresses## Although minnie's default ip address is minnie.v1kxwt, we need to use a# `real' Internet address for the ec0 connection. We also use minnie's ampr# ip  address  for  encapsulation.ifc  ec0    ipa  minnie.cs.adfa.oz.auifc  ec0    broadcast  131.236.20.255ifc  ec0    mtu  1500ifc  ax0    ipa  minnie.vk1xwt.ampr.orgifc  ax0    mtu  576ifc  ax0    broadcast  44.136.7.255ifc  encap  ipa  minnie.vk1xwt.ampr.orgifc  encap  mtu  576

With  the  above  done,  the  only  important  thing  to  do  next  is  to set  up  the  routes  for  packets.   However,  I  usually  describe  the name  servers  that  are  available  for  domain  name  to  IP  address  lookups.  This  was  before  I  found  out  that  NOS  doesn't  use  name  servers  while in  the  initialization  file.You  can  specify  several  domain  name  servers.   There  are  two  local domain  servers  at  ADFA,  which  are  specified,  as  well  as  the  authoritative name  server  for  the  AMPRnet,  UCSD.EDU.   A  maximum  lookup  wait  of 30  seconds  is  specified.MINNIE  also  acts  as  a  caching  name  server  itself:   name server requests  to  MINNIE  are  passed  on  to  the  name servers  MINNIE  knows, and  the  result  is  kept  in  MINNIE's  memory  in  case  it  is  again  requested in  the  future.   MINNIE  caches  up  to  200  domain  name  results.

##         Domain  Servers#domain  cache  wait  30000domain  cache  size  200domain  cache  clean  offdomain  addserver  ucsd.edudomain  addserver  ccadfa.cc.adfa.oz.audomain  addserver  cspyr1.cs.adfa.oz.audom  maxwait  30dom  startdom  trans  on

Finally,  routing!   A  gateway  needs  the  following  routes:

• A  route  to  the  local  AMPRnet  subnet.• A  route  to  the  local  LAN  subnet.• A  default  route  via  the  local  Internet  gateway.• Lots  of  encap  routes  to  other  AMPRnet  gateways.

Note  that  the  default  route  is  out  to  the  Internet.   This  is  done so  that  mistakes  hit  the  Ethernet  and  not  the  airwaves.   The  first three  routes  are  usually  static,  and  are  included  in  the  initialization file.   The  encap  routes  frequently  change,  and  so  MINNIE  loads them  from  a  second  file,  ENCAP.TXT.   This  file  can  be  generated by  extracting  only  those  lines  with  the  word  `ENCAP'  from  the  `GATEWAYS.XXX' file.   You  must  remember  to  remove  the  encap  route  to  your  own gateway  from  ENCAP.TXT,  otherwise  weird  things  will  happen.  Routes  are  specified  as  private.   This  prevents  your  gateway from  advertising  routes  to  the  Internet,  in  the  unlikely  event that  you  are  running  RIP.

### ##         Routing##  The default route is via ec0.  ax0  takes

### #  the  subnet 44.136.7.128/25. Other#  routes  are  via  IP  encapsulation to

### #  other 44. networks via the Internet.#  See  the file  `gateways.xxx'  for  more

#  details.  We now load from a 2nd  file,#  encap.txt;  this  makes  altering  much  easier.#route  addprivate  default            ec0  cs`gate.cs.adfa.oz.auroute  addprivate  131.236.20/24    ec0route  addprivate  44.136.7.128/25  ax0source  encap.txt

With  the  routing  specified,  MINNIE's  packet  filtering  (IP  access) and  connection  filtering  (TCP  access)  are  given.   The  rule  of  thumb here  is  to  first  specify  the  range  of  address  that  are  permitted access,  then  specify  the  range  of  addresses  denied.   If  no  denials are  specified,  then  the  default  is  to  permit  all.The  IP  access  is  simple.   All  AMPRnet  addresses  can  route  packets over  the  ax0  interface,  but  all  other  addresses  are  denied.For  the  TCP  access,  you  must  decide  which  services  should  be denied  and  from  whom.   On  MINNIE,  only  the  converse  server  running on  TCP  port  3600  has  access  denied.   TCP  access  is  permitted  on AMPRnet  addresses  and  ADFA  addresses  on  TCP  ports  1  to  3600.   TCP access  to  port  3600  is  denied  to  all  others.   Thus,  all  addresses are  implicitly  allowed  to  connect  to  all  ports  but  3600.

##    IP  Access  -  prevent  Joe  Internet  from  accessing  ax0#ip  access  permit  44/8  ax0ip  access  deny  all  ax0##   TCP Access  -  prevent Joe Internet from accessing the converse  server#tcp  access  permit  44/8  1  3600tcp  access  permit  131.236.20/24  1  3600tcp  access  deny  all  3600

TCP  and  IP  parameters  can  now  be  detailed.   The  IP  maximum  hop count  should  now  be  at  least  32,  as  the  Internet  is  large  enough to  need  this.   The  maximum  segment  size  in  TCP  should  be  set  to the  smallest  packet  size  (576  on  ax0  and  encap)  minus  40.

##         Set  TCP  and  IP  parameters#ip  ttl  32#tcp  mss  536tcp  window  2900tcp  irtt  10000tcp  syndata  on

With  the  interfaces  and  routes  ready,  we  can  begin  on  the  parameters of  the  gateway's  services.   These  are  all  readily  explained  in the  NOS  documentation,  and  I  will  omit  any  descriptions  here.

##         SMTP  parameters##         Some  mailing  list  recipients  are  stuffed  :-(smtp  batch  offsmtp  timer  500smtp  usemx  onsmtp  trace  0smtp  maxclients  7##         Mbox  parameters#attended  offmb  attend  offmb  tdisc  200mb  conv  off##         Ftp  parameters#ftptdisc  200ftype  bin##         Converse  Commands#conv  host  Canberra

MINNIE  is  an  interesting  gateway,  in  that  it  runs  both  a  SMTP mail  service  and  a  converse  server,  both  of  which  seem  to  suffer from  memory  hemorrhages.   This  means  that  MINNIE  is  unlikely  to stay  up  for  more  than  24  hours  before  running  out  of  memory  and hanging.   If  you  are  able  to,  I  suggest  you  off load  one  or  both of  these  services  to  a  non-gateway  machine.To  try  and  remedy  this  problem,  MINNIE  uses  the  `AT'  command in  NOS  to  exit  out  of  NOS  every  6  hours,  where  it  falls  back  to the  NOS  batch  file,  which  cleans  up  and  reruns  NOS,  reclaiming the  lost  memory.   This  is  mostly  successful,  but  under  heavy  use MINNIE  will  still  hang.

##         AT  commands#at  0001  exitat  0600  exitat  1200  exitat  1800  exit

Finally,  the  gateway's  services  are  started.

### ##         Start  Servers##  The telnet server is activated so that people coming in from other 44.

#  boxes can leave mail, and eventually see what other calls to connect to.start  echostart  discardstart  ftpstart  smtpstart  fingerstart  telnetstart  convers

Miscellaneous  stuff.   Logging  of  incoming  connections  and  server use  is  appended  to  the  NOS.LOG  file,  and  the  function  keys  F7  and F8  have  the  strings  `MEM STAT',  and  `T Ss'  attached,  so  MINNIE's operator  can  quickly  review  the  memory  status  and  the  TCP  connections to  the  gateway.

##         Misc##   Log  server  use.#   Set  up  some  function  keys.log  nos.logfkey  65  "mem  stat"fkey  66  "t  s"strace  0